Updated by Brandon Simpson 15 August 2023

EMERGENCY PROCEDURES PRIOR TO EXIT:

On the Ground and Prior to Takeoff

- Exit and assemble 100m

- Take all commands from PJM

Below 1000ft AGL

- Emergency landing position, seatbelt fastened

- Once aircraft stops, exit and assemble 100m

Emergency Bailout Reserve (1001ft to 3000ft AGL)

- Emergency bailout signal (index above head in circle)

- Clenched fist over yellow reserve handle and extend

- Follow JM commands, dive exit and deploy reserve IMMEDIATELY

Emergency Bailout Main (3001ft AGL and above)

- Emergency bailout signal (index above head in circle)

- Point to main pilot chute handle

- Follow JM commands, dive exit and deploy main within 5 seconds

Premature Deployment inside the aircraft

- Shout PILOT CHUTE and contain it

- Main Pilot Chute: Disconnect RSL, cutaway main, seatbelt, land with aircraft

NOTE: at the MFFS instructor can repack you pilot chute if curved pin is still in place

- Reserve: remove equipment, land with aircraft

- If pilot chute or parachute is pulled outside the aircraft, jumper and jumper in front exit

Altimeter Failure Prior to Exit

- Notify JM, (altimeter hand, thumb down)

EMERGENCY PROCEDURES IN FREEFALL

Jumper Towed by Equipment

- ARCH

- JM will free equipment

- Once free, check altimeter, check handles

- If unconscious, JM will pull you onboard

Collision on Exit

- Arch, gently push off with open hands

- Check altimeter, check handles

- DO NOT GRAB

Spinning

- Relax, arch, counter in opposite direction, check hands and feet

- Maintain altitude awareness

- If unable to maintain altitude awareness wave off and pull

NOTE: At MFFS during 1:1 instructor will assist you

Tumbling

- Relax, arch, chin up, check hands and feet

- Maintain altitude awareness

- If unable to maintain altitude awareness wave off and pull

NOTE: At MFFS during 1:1 instructor will assist you

Entering a Cloud or Loss of Visibility in Free-Fall

- Stop all movement

- Maintain heading

- Maintain altitude awareness

- Pull at prescribed altitude

Lost Goggles

- Make one attempt to replace goggles (keep elbows high)

- If unable to replace, squint

- Maintain altitude awareness

- If unable to maintain altitude awareness wave off and pull

Lost or Broken Altimeter

- Observe other jumpers, pull when they do

- If unable, clear, wave off and pull

NOTE: At MFFS during 1:1, instructor will notify when to pull

Rucksack Shift

- Counter

- If strap goes below knee, make one attempt to fix

- If unable to gain control or maintain altitude awareness, wave off and pull

Collision Avoidance during Free-Fall

- Lower jumper has the right of way

- Never get over the top of another jumper

- Use approved movement techniques (ie: turns, forward glide, and tracks) to get off a jumper’s back

Premature Activation of Parachute in Free-Fall

- Main: 3 ring, trim tabs, solid slider, pilot chute in tow

- Reserve: no 3 ring, no trim tabs, mesh slider, no pilot chute in tow

EMERGENCY PROCEDURES DURING DEPLOYMENT

PULL PRIORITIES

- PULL

- NEVER SACRIFICE ALTITUDE FOR STABILITY

Decision Altitude for Emergency Procedures

- Cutaway decision altitude 2500ft AGL

- NEVER cutaway below 1000ft AGL

- Below 1000ft AGL deploy reserve

Canopy Controllability Check

- Full brakes, determine flare point

- Look left, turn left 90 degrees

- Look right, turn right 90 degrees

50% Rule

- More than 50% opposite toggle to stop a turn = uncontrollable

- Less the 50% brakes fully flares = uncontrollable

- If canopy in uncontrollable, execute cutaway procedures

Cutaway Procedures

- Arch

- Counter (left hand)

- Look (identify red cutaway pillow)

- Grab

- Pull

- Throw away

- Counter (right hand)

- Look (identify yellow reserve pillow)

- Grab

- Pull

- Throw away

- Check (over right shoulder to see if pilot chute deploys)

HIGH SPEED MALFUNCTIONS

- No lift capability, Reduced decision-making time, EAAD (CYPRES 2) will activate

Missing Pilot Chute Handle

- Arch, trace thigh, sweep up to Main Pilot Chute Handle

- Make two attempts to locate handle; initial pull sequence is 1st attempt

- If unable, execute cutaway procedures

Hard Pull

- Use right elbow against pack tray as leverage

- If unsuccessful, execute cutaway procedures

Main Bridal Around Right Arm

- Rotate arm skyward, attempt to free using swimming motion

- If unsuccessful, execute cutaway procedures

Pilot Chute in tow

- Execute cutaway procedures

Pilot Chute Hesitation

- Check over right shoulder to disrupt burble

- If unsuccessful, execute cutaway procedures

Horseshoe Malfunction

- Execute cutaway procedures

Bag Lock

- Execute cutaway procedures

Spinning Malfunction

- Execute cutaway procedures

Line Over

- Canopy controllability check

Streamer/Snivel

- Pull rears risers

- Make two attempts, if slider moves below cascades perform CCC

- If unsuccessful execute cutaway procedures

LOW SPEED MALFUNCTIONS

- At least 5 cells inflated

Line Twists

- Thumbs down no risers, pull apart

- Bicycle kick

- Do not unstow brakes until twist are cleared

- Maintain altitude awareness, if unable to clear, execute cutaway procedures

Hung Slider

- Above cascades: pump rear risers, if slider remains above cascades,

- Make two attempts, if unsuccessful execute cutaway procedures

- Below cascades: full flare 3-4sec, perform CCC

Closed End Cells

- Full brake 3-4sec, let up quickly

- Repeat if cells don’t inflate

- Perform CCC

Pilot Chute over the nose

- Full brakes, let up, attempt to flip pilot chute over nose

- Perform CCC

Premature Brake Release

- Unstow the other brake

- Perform CCC

Tension Knots

- Stop spin with opposite rear riser

- Snap affected line group, two attempts

- Perform CCC

Broken Suspension Lines

- Any “A” line, two or more lines, execute cutaway procedures

- Only one line, not an “A” line, perform CCC

Rips/Tears

- Compare rate of decent to fellow jumpers, if falling faster execute cutaway procedures

- If hole is in bottom skin, perform CCC

- If hole is in top skin, execute cutaway procedures

Broken Control Line

- Steer with good toggle and opposite rear riser

- At safe altitude determine flare point with rear risers

- Flare and land with both rear risers only

Entering a Cloud

- Fly at 50% brakes

- Do not turn

- Maintain altitude and air awareness

DUAL CANOPY DEPLOYMENT EMERGENCIES

Side by Side

- Ensure both canopies are not entangled

- If unsure, assume they are

- Do not release brakes (If brakes are unstowed on one, unstow on other)

- If not entangled, left of the left of the left

- If entangled keep canopies together, steer outer canopy

- Attempt to land into the wind

- Do not flare, PLF

Biplane

- Ensure both canopies are not entangled

NOTE: if pilot chute of front canopy is visible under rear canopy consider entangled

- If unsure, assume they are

- Do not release brakes (If brakes are unstowed on one, unstow on other)

- If not entangled, left of the left of the left

- If entangled keep canopies together, steer front canopy with rear risers or use front canopy rear risers and rear canopy front risers.

- Attempt to land into the wind

- Do not flare, PLF

Down Plane

- Ensure both canopies are not entangled

- If unsure, assume they are

- Make every effort to get both canopies together and keep them there

- If not entangle pull red cutaway pillow

- Do not flare, PLF

Partially Deployed

- Do not unstow brakes

- If canopies inflate do not get entangled

- If both canopies inflate follow appropriate procedure

- If not entangled, left of the left of the left

- If reserve is in Free Bag, carefully gather suspension lines

- Firm grip on Free Bag, steer with one hand, at 15ft drop and flare with rear risers, PLF

CANOPY COLLISION EMERGENCIES

Actions to Avoid a Canopy Collision

- Steer with rear risers

- Always turn right

- Avoid body to body contact

- If imminent, spread eagle, cover handles with left arm

- Be prepared for violent impact

Actions if Entangled with a Fellow Jumper

- Stay calm, do not grab handles

- Only use positive communication

- Never say “cut away” unless intent is for other jumper to cutaway

Entanglements above 2000ft AGL (high jumper has good canopy)

- Higher jumper: clear entanglement

- Lower jumper: if cleared, canopy will re-inflate in 150-200ft

- Lower jumper: if not cleared, execute cutaway procedures

Entanglements between 2000ft-1000ft AGL (high jumper has good canopy)

- Higher jumper: Positive communication, listen for instructions from lower jumper

- Lower jumper: 2 options

  - Execute cutaway procedure

  - Land with higher jumper, jettison rucksack, PLF

Entanglements below 1000ft AGL (high jumper has good canopy)

- Higher jumper: Maintain lower jumper

- Lower jumper: land with higher jumper, jettison rucksack, PLF

Entanglements (neither jumper has good canopy)

- Higher jumper clear entanglement and has cutaway priority

- If impact with ground is imminent, both jumpers deploy reserve

- If both reserves inflate, cutaway from entanglement to prevent down plane

HAZARDOUS LANDING PROCEDURES

Tree Landings

- Avoid

- Do not lower CE, if lowered jettison

- Keep goggles and O2 mask on

- Face into the wind, attempt to land vertical with brakes, PLF

Water Landings

- Avoid

- Land near shore

- Jettison helmet, CE, weapon, O2

- Disconnect waist band and chest strap

- Face into the wind, PLF

- Release leg straps

- Swim upwind/upstream

- If trapped, follow seam to edge

Wire Landing

- Avoid

- Never fly over wires below 1000ft AGL

- Disconnect RSL, turn off O2, jettison CE

- Parallel wires

- If contact with ground is made cutaway

- If suspended remain motionless

- Do not allow anyone to touch you

High Winds

- Release one toggle, pull other hand over hand

- Run around top

- If dragged disconnect RSL pull red cutaway handle, do not complete cutaway procedures

Dust Devil

- Stay alert for erratic or swirling winds

NOTE: at MFFS dust devil marked with red smoke

- Avoid

- If unable to avoid, go to full flight in attempt to fully pressurize the canopy.  If time permits, set trim tabs to increase pressurization and rate of descent. Prepare to PLF.

- On ground: gather canopy and lay on top

- If unsuccessful disconnect RSL pull red cutaway handle

Off Drop Zone Landing

- Land into the wind, PLF

- Avoid hills, ravines, gullies

- Move to nearest road

- Radio DZSO

Obstacles

- Avoid, If unable contact both feet